4 x 1000L brewing equipment set for sale. The set has been used 3 years, it is complete and operational, available for viewing.

| Brewhouse Unit |  |
| --- | --- |
| - Mash/Lauter Tank Capacity: 500L (+25% Headspace minimum) Dimension: Φ1060mmx2800mm Interior Shell: SUS304 full welded; TH=3mm Exterior Shell: SUS304 full welded; TH=2mm 100% TIG welding with pure argon gas shield Interior Finish: Overall polishing to 0.4~0.6 μm without dead corner Insulation: Rock wool; TH=80mm Heating: Steam Dished top and sloped bottom Stainless steel top mounted manway Side mounted grain outdoor with grain chute 360° coverage CIP spraying ball Grain rake with VFD controlled motor and turbine reducer Easy clean and detachable sparging spray ring Inline wort lauter sight glass Laser cutting V-wire False Bottom/Filter Tangential whirlpool inlet 4pcs completely SUS304 heavy duty legs with leveling pad Thermowell with high accuracy temperature sensor | - Kettle Tank Capacity: 500L (+30% Headspace minimum) Dimension: Φ1060mmx2500mm Interior Shell: SUS304 full welded; TH=3mm Exterior Shell: SUS304 full welded; TH=2mm 100% TIG welding with pure argon gas shield Interior Finish: Overall polishing to 0.4~0.6 μm without dead corner Insulation: Rock wool; TH=80mm Heating: Steam Steam Jacket: Dimple plate on bottom and side Dished top and sloped bottom Stainless steel top mounted manway 360° coverage CIP spraying ball Sanitary leveling sight glass Steam vent by steam condenser with spray nozzles Thermowell with high accuracy temperature sensor 4pcs completely SUS304 heavy duty legs with leveling pad |
| - Transfer 1pcs Centrifugal Pump: 3M3/Hr; Sanitary SUS304 Centralized hard-piped manifold for liquid flow controlling Hard piping: Sanitary SUS304; Self-draining Inline sight glass for checking wort clarity and color Hose: Reinforced food grade; 150℃ resistance; odorless, tasteless Complete valves, fittings and all parts Every weld to a smooth sanitary finish Stainless steel multiply-position lever handle butterfly valve | - Work platform Stainless-steel setup Non slip metal floor Integrated stairs Solid and ground handrail Every weld to a smooth sanitary finish |
| - Heat exchanger Function: Wort cooling Two stages for glycol water and cold water Completely 304 SS Corrugated plates Full detachable for regular thorough cleaning Thermometer for wort outlet temperature controlling Effective Cooling Area: 6M2 Design/Test Pressure: 10bar/13bar Working Temperature: <150℃ Sanitary wort aeration mounted with stone City water, glycol water, wort inlet and outlet Dimension: 380x225x530mm |  |
| Steam Generator |  |
| Electric and automatic steam generator MAXI 60 Available steam production 30Kw Water feed can be connected directly to the water mains Steam output 40 kg/h Overall dimensions 900x900x780 mm Set of steam valves included |  |
| 1000L Fermentation Tank/Unitank; 4 pcs (4 x 1000L) |  |
| Function: Wort Fermentation or maturation Tank Capacity: 1000L (+25% Headspace minimum) Dimension: Φ1210mmx2550mm Interior Shell: SUS304 full welded; TH=3mm Exterior Shell: SUS304 full welded; TH=2mm 100% TIG welding with pure argon gas shield Interior Finish: Overall polishing to 0.4~0.6 μm without dead corner Insulation: Polyurethane; TH=80mm Glycol Jacket: Dimpled plate on cone and side Dished head and 60-degree cone bottom Sanitary side mounted manway CIP arm with 360° coverage CIP spraying ball CO2 blow-off arm with butterfly valve Full sanitary sampling valve Mechanical relief valve 2 bar on CIP arm Shockproof pressure gauge on CIP arm Rotating racking arm and port on cone with butterfly valve Tri clamp discharge arm with butterfly valve Thermowell with high accuracy temperature sensor 4pcs completely SUS304 heavy duty legs with leveling pad and legs support Complete valves, fittings and all parts Dry hopping port with solid end cap Level tube Carbonation port and stone |  |
| Glycol Cooling Unit |  |
| - Glycol Water Tank Function: Glycol water supply for wort cooling and fermentation cooling Tank Capacity: 1500L (+10% Headspace min) Dimension: Φ1360×1900mm Interior Shell: SUS304 full welded; TH=3mm Exterior Shell: SUS304 full welded; TH=2mm 100% TIG welding with pure argon gas shield Interior Finish: Overall polishing to 0.4~0.6 μm without dead corner Insulation: Polyurethane; TH=80mm Insulated flat lid cover on top and sloped bottom Thermowell with high accuracy temperature sensor Discharge arm with butterfly valve 4pcs completely SUS304 heavy duty legs with leveling pad Glycol water inlet and outlet Includes 1500L glycol water mix | - Chiller Function: Glycol water chilling 2sets Cooling Capacity: 6600Kcal/3HP Installed Power: 3.3kw Refrigerant: Freon R404A, R410A Efficient fans Stainless steel pump for glycol water recycle Protector for pressure, overload, anti-freeze, delay, overtemperature, flow Size: 850x720x1325mm |
| - Glycol Water Transfer Function: Glycol water circulation 1pcs Centrifugal Pump: 3M3/Hr; Sanitary SUS304 Standard PPR glycol piping loops Matched connection piping fittings Insulation foam for supply and return loops Solenoid valves for glycol inlet to tanks |  |
| Controlling Unit |  |
| Function: Brewery controlling Cabinet Material: Anti-power Plastic painting Instrument panel Assembling Style: Floor Type/Wall hanging Electrical Parts: CE certificated Mashing Pump on/off included VFD Rake or Agitator motor on/off included VFD Temperature display and Auto/Manu heating Control for brewhouse tanks Auto/Manu Glycol Water Pump control Auto/Manu Chiller control Temperature display and Auto/Manu Control for FVs Temperature display and Control for GWT and CLT |  |
| CIP Cart Unit |  |
| - Caustic Tank Function: Caustic liquid heating and mixing Tank Capacity: 100L Dimension: Φ500×1150mm Shell: SUS304 full welded; TH=2mm Interior Finish Polish: 0.4~0.6 μm Caustic tank with 4kw electric heating element Cone top and bottom Top glass porthole Valves and fittings included | - Sterilization Tank Function: Sterilization liquid mixing Tank Capacity: 100L Dimension: Φ500×1150mm Shell: SUS304 full welded; TH=2mm Interior Finish Polish: 0.4~0.6 μm Cone top and bottom Top glass porthole Valves and fittings included |
| - Liquid Transfer Portable cart with wheels and handle 1pcs Fixed Pump: 3M3/Hr; 0.75kw Electromagnetic starter for pump on/off Hose: Reinforced food grade; 150℃ resistance; Odorless, tasteless Hard piping: Sanitary SUS304; Self-draining Every weld to a smooth sanitary finish Complete circulation piping and fitting |  |